Colophon

| Project name | Site Studies Hollandse Kust (west) Wind Farm Zone |
| --- | --- |
| Version | 22 March 2021, HKW appendix |
| File name | HKW_20210322_PSD_Appendix C_Boundaries and Coordinates – Version March 2021 |
| Project manager | Peter-Paul Lebbink (RVO) |
| Annex(es) Note | HKW_20210322_RVO_Appendix C_Coordinates (.xlsx) Please note the coordinates in this document are complete and final and will be published in updated draft Wind Farm Site Decisions for HKWWFS VI and VII |
| Authors | Stijn Wijdeven (RHDHV-NL) Tom Moekotte (RHDHV-NL) Geert van Oorschot (RHDHV-NL) |
| Reviewed | Camiel van der Hout (EZK) Thomas Collette (RWS) Rick van der Heijden (RVO) Matté Brijder (RVO) Peter-Paul Lebbink (RVO) Alfred Dekkers (TenneT) Kees Vlak (RWS) |
| Approved | Matté Brijder (RVO) |

Only the coordinates and boundaries in the final Wind Farm Site Decisions are legally binding. The coordinates in the final Wind Farm Site Decisions HKWWFS VI and VII are aligned with this memo. Please note this memo confers no rights and is provided for convenience only.

Contents

1	Introduction	4

2	Hollandse Kust (west) Wind Farm Sites VI and VII (HKWWFS VI and VII)	6

3	Planned TenneT infrastructure	16

4	Existing infrastructure	18

# Introduction

## Purpose

The purpose of this memo is to establish the exact boundaries and coordinates for the Hollandse Kust (west) Wind Farm Sites VI and VII (HKWWFS VI and VII) (in Dutch: kavel VI en VII). The geographical information is based on the data available in January 2021.

Only the coordinates and boundaries in the final Wind Farm Site Decisions (WFSD) are legally binding. The coordinates in the final WFSDs are aligned with this Appendix. Please note this memo confers no rights and is provided for convenience only.

## Horizontal datum

The geodetic system used for horizontal projections is European Terrestrial Reference System 1989 (ETRS89).

All coordinates are given in UTM projection, for which the following applies:

- Local datum European Datum (ETRS89)

- Central Meridian 3° E (Zone 31N)

The corresponding EPSG code to this horizontal datum is:

- EPSG 25831

## GIS Information

All coordinates given in this document correspond to GIS data that is made available by the Netherlands Enterprise Agency (RVO) through the MPK file of the HKWWFZ at https://offshorewind.rvo.nl/generalw.

## Coordinate numbering

The coordinates given in this document are numbered according to the system below. Coordinate groups that exceed 100 samples have been added to the accompanying Excel file: HKW_20210322_RVO_Appendix C_Coordinates

- Coordinates that determine the outer boundaries of the HKWWFZ start with the letter code WFZ (Wind Farm Zone) followed by an underscore and a number (WFZ_01, WFZ_02, …);

- Coordinates that determine the outer boundaries of the HKWWFS start with the letter code S (Site) followed by an underscore and a number (S_01, S_02, …);

- Coordinates that determine the maintenance zones of pipes and cables in the HKWWFZ start with the letter code MZ (Maintenance Zone) followed by an underscore and a number (MZ_01, MZ_02, …). These coordinates are in the separate Excel appendix with coordinates;

- Coordinates of objects which might have archaeological value start with the letter code AV (Archaeological Value) followed by an underscore and a number (AV_01, AV_02, …);

- Coordinates of objects are marked as wrecks start with the letter code W (Wreck) followed by an underscore and a number (W_01, W_02, …);

- Coordinates of magnetic anomalies possibly indicating UXO objects or objects with archaeological value start with the letter code MA (Magnetic Anomaly) followed by an underscore and a number (MA_01, MA_02, …). These coordinates are in the separate Excel appendix with coordinates;

- Coordinates that determine the location of the existing cables and pipelines in the HKWWFZ start with the letter code CP (Cables and Pipes) followed by an underscore and a number (CP_001, CP_002, …). The following number is derived from the GIS dataset (field: ObjectID). These coordinates are in the separate Excel appendix with coordinates;

- Coordinates of objects are marked as platform start with the letter code Q (Platform) followed by an underscore and a number (Q_01, Q_02, …);

- Coordinates of objects are marked as borehole start with the letter code BH (Borehole) followed by an underscore and a number (BH_01, BH_02, …);

## Orientation figures

All figures included in this document are presented with the north direction facing upwards.

# Hollandse Kust (west) Wind Farm Sites VI and VII (HKWWFS VI and VII)

Figure 2.1	Overview of the HKWWFZ – Zone

| Table 2.1 Coordinates of the HKWWFZ – Wind Farm Zone |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Wind Farm Zone of HKWWFZ |  |  |  |  |  |  |
| ETRS 1989 UTM Zone 31N EPSG 25831 |  |  |  |  |  |  |
| Point No. | Easting | Northing |  | Point No. | Easting | Northing |
| WFZ_01 | 535,872.2 | 5,814,654.0 |  | WFZ_08 | 556,364.1 | 5,831,312.2 |
| WFZ_02 | 537,022.3 | 5,822,012.5 |  | WFZ_09 | 549,503.4 | 5,823,303.3 |
| WFZ_03 | 534,999.3 | 5,822,170.2 |  | WFZ_10 | 548,730.0 | 5,822,551.6 |
| WFZ_04 | 537,570.7 | 5,825,520.7 |  | WFZ_11 | 549,065.8 | 5,821,741.1 |
| WFZ_05 | 537,761.1 | 5,826,738.8 |  | WFZ_12 | 549,721.8 | 5,820,157.5 |
| WFZ_06 | 560,375.0 | 5,853,830.4 |  | WFZ_13 | 547,496.6 | 5,820,125.9 |
| WFZ_07 | 556,100.8 | 5,831,438.1 |  | WFZ_14 | 536,741.8 | 5,815,063.4 |

## Wind Farm Sites VI and VII

Figure 2.2	Overview of the HKWWFS – Site

| Table 2.2 Coordinates of the HKWWFS – Wind Farm Site VI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Wind Farm Site VI of HKWWF |  |  |  |  |  |  |
| ETRS 1989 UTM Zone 31N EPSG 25831 |  |  |  |  |  |  |
| Point No. | Easting | Northing |  | Point No. | Easting | Northing |
| S_01 | 549,259.1 | 5,840,513.5 |  | S_04 | 556,672.6 | 5,834,433.9 |
| S_02 | 558,353.9 | 5,851,409.2 |  | S_05 | 555,572.5 | 5,832,506.4 |
| S_03 | 559,589.1 | 5,849,713.0 |  |  |  |  |

| Table 2.3 Coordinates of the HKWWFS – Wind Farm Site VII |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Wind Farm Site VII of HKWWFZ |  |  |  |  |  |  |
| ETRS 1989 UTM Zone 31N EPSG 25831 |  |  |  |  |  |  |
| Point No. | Easting | Northing |  | Point No. | Easting | Northing |
| S_06 | 539,465.9 | 5,828,781.1 |  | S_08 | 554,930.8 | 5,831,382.2 |
| S_07 | 548,473.2 | 5,839,572.0 |  | S_09 | 552,835.3 | 5,827,710.8 |

## Maintenance Zone and Coordinates

Site VI and VII have their own maintenance zones and coordinates. Therefore two overview maps are presented, one for each site. Additional zoomed in cut-outs are presented to further clarify the position of coordinates. Site VI is presented in Figure 2.3a, Site VII in Figure 2.3b. The coordinates of the Maintenance zones are in the Excel file: HKW_20210322_RVO_Appendix C_Coordinates

Figure 2.3a	Overview of the Maintenance zone of the HKWWFS – Site VI

Figure 2.3b	Overview of the Maintenance zone of the HKWWFS – Site VII

## Possible Archaeological Objects and Coordinates

Figure 2.4	Overview of the HKWWFZ – Possible Archaeological Objects

| Table 2.4 Coordinates of possible Archaeological Objects in HKWWFZ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Archaeological objects in HKWWFZ |  |  |  |  |  |
| ETRS 1989 UTM Zone 31N EPSG 25831 |  |  |  |  |  |
| Point No. | Description | Classification | Code | Easting | Northing |
| AV_01 | elongated to rectangular hard reflective object | Possible wreck | S_0039 | 536,556.0 | 5,817,013.0 |
| AV_02 | spherical object amidst a scour 13 x 11 m with 118 nT magnetic anomaly | Possible wreck | S_0093 | 538,628.0 | 5,824,408.0 |
| AV_03 | two elongated objects | Possible wreck site | S_0095 | 538,755.0 | 5,824,686.0 |
| AV_04 | two elongated objects | Possible wreck site | S_0096 | 538,786.0 | 5,824,717.0 |
| AV_05 | high reflective exposure | Possible buried wreck | S_0336 | 544,748.0 | 5,823,694.0 |
| AV_06 | high reflective exposure | Possible buried wreck | S_0353 | 547,417.0 | 5,836,653.0 |
| AV_07 | contours of a ship-like object | Buried ship wreck | S_0401 | 544,499.0 | 5,821,369.0 |
| AV_08 | cluster of contacts | Possible wreck site | S_0412 | 544,989.0 | 5,819,800.0 |
| AV_09 | cluster of contacts | Possible wreck site | S_0413 | 544,995.0 | 5,819,792.0 |
| AV_10 | linear contact amidst cluster of contacts | Possible buried wreck | S_0679 | 553,839.0 | 5,842,543.0 |
| AV_11 | Wreck DHY 2247. Duikteam Zeester: Biarritz, sunk 1940 | Wreck Biarritz | NCN2056 | 540,645.0 | 5,828,700.0 |
| AV_12 | Wreck DHY 2256. Duikteam Zeester: SS Paris, sunk 1939 | Wreck SS Paris | NCN2064 | 540,162.0 | 5,829,452.0 |
| AV_13 | Distributed remains of wreck DHY 2284 | Wreck | NCN2091 | 551,689.0 | 5,838,477.0 |
| AV_14 | Wreck DHY 2292. Duikteam Zeester: Boezemwrak close to platform | No visible contacts, sandwave, mag. anomaly 2031 nT | NCN2098 | 554,783.0 | 5,842,860.0 |
| AV_15 | Wreck DHY 2468. Unknown wreck reported 1984 | Wreck | NCN2250 | 548,149.0 | 5,832,487.0 |
| AV_16 | Wreck debris | Wreck, partly buried | NCN2469 | 555,444.0 | 5,845,242.0 |
| AV_17 | Wreck DHY 3427. Unknown wreck reported with sonar 1997 | Possible wreck | NCN2809 | 554,440.0 | 5,845,409.0 |
| AV_18 | Wreck DHY 2255, Unknown wreck found 1970 | No visible contacts | NCN2063 | 540,648.0 | 5,829,062.0 |
| AV_19 | Wreck DHY 2283. Unknown wreck found 1946 | No visible contacts | NCN2090 | 549,558.0 | 5,838,909.0 |
| AV_20 | Wreck DHY 2291 Unknown wreck found 1961 | No visible contacts | NCN2097 | 551,880.0 | 5,843,043.0 |
| AV_21 | Wreck DHY 2294. Mast reported 1898, not confirmed since | No visible contacts | NCN2100 | 558,429.0 | 5,842,871.0 |
| AV_22 | Wreck DHY 3498 | No visible contacts | NCN2844 | 553,958.0 | 5,830,158.0 |
| AV_23 | Wreck DHY 3500. Wreck debris reported 2014 | No visible contacts | NCN2845 | 554,572.0 | 5,833,117.0 |
| AV_24 | Wooden wreck remains, discovered in 2002. | No visible contacts | NCN9226 | 556,213.0 | 5,832,620.0 |

A buffer zone of 100 m should be applied around the few objects found which have yet to be determined to have no archaeological value, thereby prohibiting any activities in their locations. This also applies to cable trenching and anchorages of work vessels. Please note that the buffer zone may be reduced if it can be substantiated that the applied activity and disturbance has no effect on the (possible) archaeological object.

## Wrecks with possible Archaeological Value in HKWWFZ

Figure 2.5	Overview of the Wrecks in the HKWWFZ

| Table 2.5 Wrecks with possible archaeological value in HKWWFZ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Archaeological objects in HKWWFZ |  |  |  |  |  |
| ETRS 1989 UTM Zone 31N EPSG 25831 |  |  |  |  |  |
| Point No. | NCN | Type | Description | Easting | Northing |
| W_01 | 439 | Wreck | Wreck yacht Regina, 13x3x2 meter, sunk 01-05-2008 after collision | 548,806.0 | 5,821,993.0 |
| W_02 | 522 | Wreck | Wreck DHY 522. Duikteam Zeester: Wreck of coast vessel, sunk 1980, standing right up, superstructure is gone, close by platform | 549,026.0 | 5,823,162.0 |
| W_03 | 2035 | Wreck | Wreck DHY 2221. Unknown small wreck found 1959, not confirmed since | 550,310.0 | 5,822,477.0 |
| AV_11 | 2056 | Wreck | Wreck DHY 2247. Duikteam Zeester: "Paaswrak 1", close to the Brown bank | 540,651.0 | 5,828,702.0 |
| W_04 | 2057 | Wreck | Wreck DHY 2248. Duikteam Zeester: Wreck Dutsch fishing trawler Stellendam 4, sunk 1969 | 550,864.0 | 5,827,791.0 |
| AV_18 | 2063 | Wreck | Wreck DHY 2255, Unknown wreck found 1970 | 540,648.0 | 5,829,062.0 |
| AV_12 | 2064 | Wreck | Wreck DHY 2256. Duikteam Zeester: Norwegian cargo ship Biarritz from 1920, sunk 1940 | 540,173.0 | 5,829,482.0 |
| AV_19 | 2090 | Wreck | Wreck DHY 2283. Unknown wreck found 1946 | 549,558.0 | 5,838,909.0 |
| AV_13 | 2091 | Wreck | Distributed remains of wreck DHY 2284 | 551,689.0 | 5,838,477.0 |
| AV_20 | 2097 | Wreck | Wreck DHY 2291 Unknown wreck found 1961 | 551,880.0 | 5,843,043.0 |
| AV_14 | 2098 | Wreck | Wreck DHY 2292. Duikteam Zeester: Boezemwrak close to platform | 554,776.0 | 5,842,849.0 |
| AV_21 | 2100 | Wreck | Wreck DHY 2294. Mast reported 1898, not confirmed since | 558,429.0 | 5,842,871.0 |
| W_05 | 2110 | Wreck | Wreck DHY 2304. Wreck reported 1946, not confirmed since | 550,906.0 | 5,844,640.0 |
| W_06 | 2120 | Wreck | Wreck DHY 2315. Duikteam Zeester: Submarine Doris | 555,194.0 | 5,849,035.0 |
| AV_15 | 2250 | Wreck | Wreck DHY 2468. Unknown wreck reported 1984 | 548,152.0 | 5,832,498.0 |
| AV_16 | 2469 | Wreck | Wreck debris | 555,440.0 | 5,845,241.0 |
| AV_17 | 2809 | Wreck | Wreck DHY 3427. Unknown wreck reported with sonar 1997 | 554,452.0 | 5,845,413.0 |
| W_07 | 2810 | Wreck | Distributed remains of wreck | 535,978.0 | 5,821,107.0 |
| AV_22 | 2844 | Wreck | Wreck DHY 3498 | 553,958.0 | 5,830,158.0 |
| AV_23 | 2845 | Wreck | Wreck DHY 3500. Wreck debris reported 2014 | 554,572.0 | 5,833,117.0 |
| AV_24 | 9226 | Wreck | Wooden wreck remains, discovered in 2002. ARCHIS wng 47163 | 556,213.0 | 5,832,620.0 |
| W_08 | 9299 | Submarine | French submarine Doris, sunk mei 1940, cannon salvaged in 2003. ARCHIS wng 48181 | 555,298.0 | 5,849,442.0 |
| W_09 | 15219 | Wreck | Norwegian cargo vessel Nordfrakt, sunk 25-10-1992, dimensions 76x12x2m. RWS SR 11968 | 555,554.0 | 5,833,512.0 |

Whilst installing wind turbines and the various infield and export cables, developers are advised to avoid areas where wrecks have been identified, by implementing a 100 m buffer zone.

## Magnetic Anomalies

Figure 2.6	Overview of the HKWWFZ –	Magnetic Anomalies (possible buried ferrous objects)

Whilst installing wind turbines and the various infield and export cables, developers are advised to avoid areas where magnetic anomalies have been identified, again by implementing a 100 m buffer zone. The coordinates of the magnetic anomalies is in the Excel file : HKW_20210322_RVO_Appendix C_Coordinates

# Planned TenneT infrastructure

Figure 3.1	Planned TenneT Platform and Cables, the export and redundancy cable routes and the infield cable corridors in the HKWWFZ. The labels correspond with the planned TenneT Cables.

The positioning of the jacket (and thereby the J-tubes) is dependent on the design of the platform. Currently, the contractor of the platform, and therefore the exact design is not yet known. Therefore the infield cable corridor in the safety zone (500m) should be regarded as a guideline. In accordance with TenneT and the OWF developers the exact routes of these infield cables will be planned.

The cables to the platforms are currently not yet planned but will be positioned within the Maintenance Zones (see Figure 2.2 and 2.3).

## TenneT Planned Substations

| Table 3.1 Coordinates of the Planned TenneT Platforms HKW in the HKWWFZ |  |  |
| --- | --- | --- |
| Planned TenneT Platform HKW |  |  |
| ETRS 1989 UTM Zone 31N EPSG 25831 |  |  |
| Point No. | Easting | Northing |
| TOS_1 | 554,395.1 | 5,836,951.8 |
| TOS_2 | 549,905.6 | 5,829,544.0 |

# Existing infrastructure

## Cables and pipelines

Due to ocean currents cables and pipes might gradually move on the sea bed or get covered by sand. Therefore the ‘as laid’ cables and pipes are not always the same as the ‘as found’ cables and pipes by the survey. It is advised to consult both datasets in the planning of future wind turbines. Figures 4.1a and 4.1b depict the as found and as laid cable information. The coordinates of the as found cables and pipes is in the Excel file : HKW_20210322_RVO_Appendix C_Coordinates

Figure 4.1a 	Active and inactive cables and pipelines (as laid) in and near the HKWWFZ. The labels  correspond with the cables and pipes in tables 4.1a and 4.1b.

There are several existing cables and pipelines (both active and abandoned) crossing the HKWWFZ.

| Table 4.1a Existing cables in the HKWWFZ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cables in use |  |  |  |  |
| No. | Name | Route | Type | Status |
| 1 | Atlantic Crossing 1 Segment B1 | Castricum (NL) naar Whitesand (GB) | Telecom | In use |
| 2 | PANGEA Segment 2 | Lowesoft (GB) naar Egmond (NL) | Telecom | In use |
| 3 | UK – NL 14 | Egmond (NL) naar Winterton (GB) | Telecom | Abandoned |
| 4 | Rembrandt 1 | Beverwijk (NL) Lowesoft (GB) | Telecom | Abandoned |
| 10 | UK – NL 10 | Egmond (NL) naar Lowesoft (GB) | Telecom | Abandoned |
| 11 | Rioja 3 | Veurne (B) naar Egmond (NL) | Telecom | Abandoned |

| Table 4.1b Existing pipelines in the HKWWFZ |  |  |  |
| --- | --- | --- | --- |
| Pipelines in use |  |  |  |
| No. | Route | Type | Status |
| 5 | P12-SW to P6-A | Glycol | Active |
| 6 | P9-Horizon-A to Q1-Helder-AW | Oil | Active |
| 7 | Q4-A to P6-A | Gas | Active |
| 8 | P6-D to P6-B | Gas | Active |
| 9 | P9-B to P6-D | Gas | Active |
| 12 | P6-B to P6-A | Gas | Active |
| 13 | P6-C to P6-B | Glycol | Abandoned |
| 14 | P6-S to P6-B | Gas | Abandoned |

Figure 4.1b Active and inactive cables and pipelines (as found) in and near the HKWWFZ. The labels  correspond with the cables and pipes in tables 4.1a and 4.1b.

## Existing Platforms and boreholes

There are several existing platforms and boreholes (both active and inactive) in or around the HKWWFZ. Boreholes have been grouped when sharing the same coordinates. The coordinates of the existing platforms can be found in Table 4.2c, the coordinates boreholes in Table 4.2d.

Figure 4.2	Existing platforms and boreholes in and near the HKWWFS

| Table 4.2a Existing Platforms |  |  |  |
| --- | --- | --- | --- |
| Existing Platforms |  |  |  |
| Point No. | Name | Type | Status |
| Q_01 | P6-A | Platform | Existing |
| Q_02 | P6-B | Platform | Existing |
| Q_03 | P6-C | Platform | Removed |
| Q_04 | P6-D | Platform | Existing |
| Q_05 | P09-HORIZON | Platform | Existing |

| Table 4.2b Boreholes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Existing Boreholes |  |  |  |  |  |  |
|  | UWI | Borehole code | NITG number | Owner | Client | Status |
| BH_01 | 7417 | P06-01 | BP060183 | Wintershall | Mobil | Suspended |
| BH_02 | 7419 | P06-03 | BP060172 | Wintershall | Mobil | Producing/Injecting |
|  | 7619 | P06-B-01 | BP060171 | Wintershall | Mobil | Closed-in |
|  | 7777 | P06-B-03 | BP060173 | Wintershall | Mobil | Closed-in |
|  | 8339 | P06-B-04 | BP060174 | Wintershall | Clyde | Suspended |
| BH_03 | 7431 | P08-02 | BP080048 | Mobil | Mobil | Abandoned |
|  | 9131 | P08-03 | BP080088 | Mobil | Mobil | Failure/sidetracked |
|  | 9132 | P08-03-S1 | BP080089 | Mobil | Mobil | Failure/sidetracked |
|  | 7432 | P08-03-S2 | BP080049 | Mobil | Mobil | Abandoned |
| BH_04 | 7434 | P09-02 | BP090238 | TAQA | Amoco | Closed-in |
|  | 7544 | P09-04 | BP090240 | TAQA | Amoco | Abandoned |
|  | 7655 | P09-05 | BP090241 | TAQA | Amoco | Closed-in |
| BH_05 | 7535 | P08-05 | BP080051 | Mobil | Mobil | Abandoned |
|  | 7527 | P08-05-S1 | BP080052 | Mobil | Mobil | Abandoned |
|  | 7528 | P08-05-S2 | BP080053 | Mobil | Mobil | Abandoned |
| BH_06 | 8531 | P06-C-01 | BP060175 | Wintershall | Mobil | Failure/sidetracked |
|  | 8529 | P06-C-01-S1 | BP060177 | Wintershall | Mobil | Failure/sidetracked |
|  | 7694 | P06-C-01-S2 | BP060176 | Wintershall | Mobil | Closed-in |
|  | 7736 | P06-C-02 | BP060178 | Wintershall | Mobil | Closed-in |
| BH_07 | 7781 | P09-HORIZON-A-01 | BP090245 | Chevron | Unocal | Failure/sidetracked |
|  | 8452 | P09-HORIZON-A-01-S1 | BP090246 | Chevron | Unocal | Closed-in |
|  | 8152 | P09-HORIZON-A-02 | BP090247 | Chevron | Unocal | Producing/Injecting |
|  | 8153 | P09-HORIZON-A-03 | BP090248 | Chevron | Unocal | Plugged back and sidetracked |
|  | 8166 | P09-HORIZON-A-03-S1 | BP090257 | Chevron | Unocal | Producing/Injecting |
|  | 8154 | P09-HORIZON-A-04 | BP090249 | Chevron | Unocal | Producing/Injecting |
|  | 8155 | P09-HORIZON-A-05 | BP090250 | Chevron | Unocal | Plugged back and sidetracked |
|  | 8162 | P09-HORIZON-A-05-S1 | BP090251 | Chevron | Unocal | Producing/Injecting |
|  | 8167 | P09-HORIZON-A-06 | BP090252 | Chevron | Unocal | Producing/Injecting |
|  | 8204 | P09-HORIZON-A-07 | BP090253 | Chevron | Unocal | Plugged back and sidetracked |
|  | 8450 | P09-HORIZON-A-07-S1 | BP090254 | Chevron | Unocal | Producing/Injecting |
|  | 8201 | P09-HORIZON-A-08 | BP090255 | Chevron | Unocal | Plugged back and sidetracked |
|  | 8451 | P09-HORIZON-A-08-S1 | BP090256 | Chevron | Unocal | Closed-in |
| BH_08 | 8490 | P06-10 | BP060181 | Wintershall | Clyde | Abandoned |
|  | 7997 | P06-S-01 | BP060180 | Wintershall | Mobil | Abandoned |
| BH_09 | 8069 | P09-07 | BP090242 | Chevron | Unocal | Abandoned |
| BH_10 | 8245 | P08-06 | BP080054 | ENGIE E&P Nederland B.V. | Occidental | Abandoned |
| BH_11 | 8465 | P06-D-01 | BP060179 | Wintershall | Clyde | Producing/Injecting |
| BH_12 | 9044 | P08-07 | BP080084 | Wintershall | Wintershall | Abandoned |

| Table 4.2c Coordinates for existing Platforms |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Existing Platforms |  |  |  |  |  |  |
| ETRS 1989 UTM Zone 31N EPSG 25831 |  |  |  |  |  |  |
| Point No. | Easting | Northing |  | Point No. | Easting | Northing |
| Q_01 | 551,031.4 | 5,845,314.0 |  | Q_04 | 549,013.4 | 5,839,204.0 |
| Q_02 | 554,262.4 | 5,843,360.0 |  | Q_05 | 550,242.0 | 5,822,755.0 |
| Q_03 | Removed | Removed |  |  |  |  |

| Table 4.2d Coordinates for Boreholes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Existing Boreholes |  |  |  |  |  |  |
| ETRS 1989 UTM Zone 31N EPSG 25831 |  |  |  |  |  |  |
| Point No. | Easting | Northing |  | Point No. | Easting | Northing |
| BH_01 | 556,359.0 | 5,842,226.0 |  | BH_07 | 550,242.0 | 5,822,755.0 |
| BH_02 | 554,264.0 | 5,843,361.0 |  | BH_08 | 552,833.0 | 5,836,933.0 |
| BH_03 | 544,221.0 | 5,826,065.0 |  | BH_09 | 550,671.0 | 5,831,258.0 |
| BH_04 | 548,758.0 | 5,823,713.0 |  | BH_10 | 541,883.0 | 5,823,229.0 |
| BH_05 | 543,133.0 | 5,826,296.0 |  | BH_11 | 549,015.0 | 5,839,204.0 |
| BH_06 | 555,958.0 | 5,841,313.0 |  | BH_12 | 542,467.0 | 5,831,946.0 |